PostVars List for LazyCam post processor:

MODAL Vars

To make the modal settings change only when the value changes use the []’s. Here is an example of  how to use the []’s:

[ G{MODE}] This will only output the G{MODE} if it is not the same as the last one that it output.

{MODE}  - Will set the mode that the move or cycle should be done in.

Example: G{MODE} Output  G0

{XPOS} –Will give the endpoint in the X axis of the move that is being done.

Example: X{XPOS} Output  X12.3456

{YPOS} – Will give the endpoint in the Y axis of the move that is being done.

Example: Y{YPOS} Output  Y12.3456

{ZPOS} – Will give the endpoint in the Z axis of the move that is being done.

Example: Z{ZPOS} Output  Z12.3456

{IPOS} – Will give the center point of an arc in ABS or INC depending on the

setting on the “Post Options” Dialog.

Example: I{IPOS} Output  I12.3456

{JPOS} – Will give the center point of an arc in ABS or INC depending on the

setting on the “Post Options” Dialog.

Example: J{JPOS} Output  J12.3456

{FEED} – Will put out the feed rate that is needed at the time. For moves in the X,Y

CutFeed will be used and for Z moves Plunge Feed will be used.

Example: F{FEED} Output  F1234.56

{RAPIDHEIGHT}  - Will give you the Rapid height of the chain that is being

posted.

Example: Z{RAPIDHEIGHT} Output  Z12.3456

{STARTHEIGHT}  - Will give you the Start height of the chain that is being

posted.

Example: Z{STARTHEIGHT} Output  Z12.3456

{SPINDLEON}  - Will output the direction and speed of the spindle.

Example: {SPEEDON} Output M3 S1234

{SPINDLEOFF}  - Will output the direction and speed of the spindle.

Example: {SPEEDOFF} Output M5

Non Modal Vars

{TOOLNUM} – Will Give the Tool Number of the chain that is going to be cut.

Example: T{TOONUM} Output T1

{COOLANT} – Will turn the coolant on if it is enabled in the “Post Options” dialog

Example: {COOLANT} Output M8

{PIERCEDELAY} – Will insert the code to delay the pierce for Plasma. The time is set

in the Plasma settings under “Post Options”.

Example: { PIERCEDELAY } Output G4 P123

{LIFTDELAY} – Will insert the code to delay the lift for Plasma. The time is set

in the Plasma settings under “Post Options”.

Example: {LIFTDELAY} Output G4 P124